DS KEYBOARD TECHNIC GmbH

User’s Manual: W30 -60 - 90 keyboard systems

General:

The W30-W60-W90 type keyboard systems produced by DS Keyboard Technic are freely programmable keyblocks with an IBM compatible interface. These keyboards are also available with a serial RS 232 interface. Programming is done by a TEACH program and a PC.

All keys can be programmed with a single code or strings up to 30 digits. In case of the serial versions, ASCII-codes can be input as a clear text, as a decimal code or a hexadecimal code.

Maximum memory 16 KB.

System configuration:      IBM compatible PC with colour monitor and mouse.

The TEACH program is available in the following languages: German (D), English (E) and

French (F).

How it is done:

Put in the disk in „a“ drive and call up the different versions as follows:

Version 30: TEACH  blank  language (D, E, or F)  blank  30

Version 60: TEACH  blank  language (D, E, or F)  blank  60

Version 90: TEACH  blank  language (D, E, or F)  blank  90

Now you can see a standard MF2 keyboard layout on the screen. Underneath you can see the matrix of the keyboard to be programmed.

1. Complete new installation

Put the cursor on the position to be programmed. Click the left mouse key (the field changes the colour) and move the cursor on the symbol of the standard keyboard which you wish to program. Click on this symbol with the left mouse key so that it is tranferred to the matrix. If you wish to transfer a string do as before. But in this case you will have to select the symbols with the right mouse key up to the final symbol. Then complete the programming with the left mouse key. The whole string appears once again under the matrix as a clear text.

In this way the complete matrix can be programmed.

2. Storage

If  you wish to store the prepared table, click on the field Storage. Put in a name and store the table.

3. Loading

If you wish to use a stored table, click on Loading and give in the name of the table.

On your disk the file name Standard is already stored.

4. Programming

If the desired table is finished, click on Programming and follow the user’s instructions. Put in the keyboard to be programmed instead of the standard keyboard and click on the left mouse key. The programming will run (about 80 sec.) After programming, leave the Teach program via ESC. Then draw the keyboard plug out and once again in to reset the keyboard.

Now the programmed keyboard is ready for work.

Optionally: Serial Version

In case of the serial version the keyboard is provided with a 25 positon D Sub socket. The keyboard is connected to COM 2 of the PC. Distribution voltage (+ 5V) is effected via an adapter that is plugged in between PC and standard keyboard.

How it is done:

Put in the disk in „a“ drive and call up the S-TEACH program. Now the matrix of the keyboard to be programmed appears and next to it a column with the numbered keys 1 to 30.

Go with the arrow key to the key number  you wish to program. Open the table with ENTER. Now you can put in ASCII symbols as a clear text or also as decimal or hexadecimal values. Both have to be marked with the symbols < >.

Example:     letter A       clear text        A

decimal          <65>

hexadecimal  <41>

Programming will be done by the program Programming .

Connector assignment IBM version:        5 position DIN plug

Pin 1       Clock

Pin 2       Data

Pin 4       Gnd

Pin 5       + 5V

Connector assignment RS 232 version:    25 position socket

Pin 2        RxD

Pin 3        TxD

Pin 7         Gnd

Pin 15       + 5V

Pin 4

Pin 5

Pin 6

Pin 8

Pin 20

5 position DIN plug

Pin 4          Gnd

Pin 5          + 5V

DS Keyboard Technic GmbH Herrenlandstraße 31 D-78315 Radolfzell

Tel. 0049/ 7732/8000-0 Fax 0049/7732/8000-44

MAN_W30.DOC